Scanner configuration/information

Agile X IS

Type 17-A1B*-1…/…….

Revision 0 / Status: 12th March 2019

Reservation: We reserve the right to make technical modifications. Changes, errors and misprints shall not justify any claim for damages.

# Table of contents

Table of contents	2

1.	Built in scan engine	3

1.1	Decode Distances	3

1.2	Decode Zone	4

1.3	Decodable barcode types	4

2.	Configuration options	5

2.1	HotTab Tool	6

2.2	MotoSetting v1.0.2 Tool	7

3.	Configuration samples for HotTab	11

3.1	FNC1	11

3.2	Replace characters	12

3.3	Prefix & Suffix	13

# Built in scan engine

In Agile X IS series is the Zebra 1D/2D Imager SE4500-SR built in.

The scan engine is switched on/off and basic configured over HotTab tool.

The scan engine is internal connect thru serial interface COM port 15.

## Decode Distances

| Symbol density/ Barcode type | Barcode content / Contrast1 | Typical working ranges |  |
| --- | --- | --- | --- |
|  |  | Near | Far |
| - 5.0 mil Code 39 | - ABCDEFGH 80% MRD | - 2.1 in 5.33 cm | - 7.5 in 19.05 cm |
| - 6.67 mil PDF417 | - 4 Col, 20 Rows 80% MRD | - 3.4 in 8.64 cm | - 7.1 in 18.03 cm |
| - 7.5 mil Code 39 | - ABCDEF 80% MRD | Note 2 | - 10.6 in 26.92 cm |
| - 10 mil PDF417 | - 3 Col, 17 Rows 80% MRD | Note 2 | - 10.1 in 25.65 cm |
| - 13 mil UPC-A | - 012345678905 80% MRD | - 1.6 in 4.06 cm | - 15.5 in 39.37 cm |
| - 15 mil PDF417 | 80% MRD | Note 2 | - 14.7 in 37.34 cm |
| - 15 mil Data Matrix | - 18 x 18 Modules 80% MRD | - 2.8 in 7.11 cm | - 12.4 in 31.50 cm |
| - 20 mil Code 39 | - 123 80% MRD | Note 2 | - 24.7 in 62.74 cm |

|  | Note 1) Contrast is measured as Mean Reflective Difference (MRD) at 670 nm. |
| --- | --- |
|  | Note 2) Near distances are field-of-view (FOV) limited. |
|  | Note 3) Working range specifications at temperature = 23°C, pitch=18°, roll=0°, skew=0°, photographic quality, ambient light ~30 ft-c, humidity 45-70% RH. |
|  | The scan engine minimum and maximum ranges depend on the bar code symbology/type and the module width (in mil) used. |
|  | The decode range depends on the quality and size of the bar code and on the scan engine’s software setting. |
|  | The scan engines used comply with laser class CDRH Class II / IEC 825 Class 2 |

## Decode Zone

The following figure shows the decode zone for the SE4500-SR. Typical values and distances for selected barcode densities listed in table in chapter 1.1.

The minimum element width (or “symbol density”) is the width in mils of the narrowest element (bar or space) in the symbol.

- Note: - The decode zone is from Zebra integration guide for SE4500 scan engine. - Typical performance at 73°F (23°C) on high quality symbols in normal room light. Vcc = 3.3V

* Minimum distance determined by symbol length and scan angle.

## Decodable barcode types

| All supported 1D-Barcodes |  | All supported 2D-Barcodes (only supported with Imager versions) |  |
| --- | --- | --- | --- |
| 1D-Symbol/Codes |  | 2D-Symbol/Codes |  |
| Code 11 | Code 39 | Aztec | Micro PDF-417 |
| Code 93 | Code 128 | Australian 4-state | Maxi Code |
| Codabar | Coupon code | Canadian 4-state | PDF-417 |
| Chinesisch 2 of 5 | Discrete 2 aus 5 | Composite AB | QR Code |
| Interleaved 2 of 5 | Trioptic 39 | Composite C | TLC39 |
| EAN-8 | EAN-13 | Data Matrix | UK 4-state |
| UPCA | UPCE | Dutch Kix | US Planet |
| UPC/EAN Supplementals | MSI | Japanese 4-state | US Postnet |
| Webcode | RSS-14 | PDF-417 Macro | USPS 4-state (US4CB) |
| RSS Limited | RSS Expanded | (Macro) Micro PDF-417 | microQR |

# Configuration options

Following configuration options are available.

| Tool | Description |
| --- | --- |
| HotTab | HotTab provides quick access to the tablet’s functions and features, such as turning on/off scan engine, Wifi, Bluetooth, GPS, programming function key, and changing power management. As Windows starts up, HotTab starts and runs in the background. To display the HotTab control utility, press the (Menu) button located on the front panel, OR launching the HotTab Utility application. The tool is used to enable/disable scan engine. The tool provides the possibility of simple configuration. |
| MotoSetting v1.0.2 | This is a tool that can be optionally installed. The tool enables the activation/deactivation of barcode types and a limited possibility of barcode-specific settings. |
| Zebra development kit | Zebra offers a development kit for the scan engine to program your own complete application. The kit includes: - SE-4500 Integration Guide - PL3307 Integration Guide (used decoder board) - Universal Installation Guide - SDK with sources and samples - The latest version is available on the support download page: - https://www.zebra.com/us/en/support-downloads/oem/oem-engines/se4500.html - Note: - This option is only for software developers that design their own application or that implement scan engine in their current application. |

## HotTab Tool

To enable the Barcode Reader feature, perform the following procedure:

Check the status colour of the Barcode icon:

| Item | Status Colour | Description |
| --- | --- | --- |
|  | Orange | Barcode Reader is enabled. Function key 2 (Fn2) is dedicated to the barcode reader. |
|  | White | Barcode Reader is disabled. |

If the Barcode Reader feature is disabled (white), tap on the icon to enable it (orange).

The physical function key 2 (Fn2) is dedicated to the barcode reader for triggering.

Enable the function and a window appears for easy activation, setup and demonstration.

| Scan engine is activated HotTab tool is set to . This means that COM15 is free and other scan application can access the scan engine. | Scan engine is activated HotTab tool is set to . This means that COM15 is idle and no other scan application can access the scan engine. |
| --- | --- |
| Scan engine can used by another application. E.g. “MotoSetting v1.0.2” to enable/disable barcode symbologies. | Scan engine can be tested in this setup. The tool allows some basic configuration as setup of prefix, suffix, replacement of characters and others. |

Select On  so that the barcode reader is activated.
Barcode data can be tested by pressing the Fn2 key. The result is displayed in the bottom field of the window.

|  | The Barcode reader is internal dedicated to COM port 15. |
| --- | --- |
|  | Important Note: For use of other tools it is important to open the window above and set it to and that the COM port 15 is free for use. Otherwise other tool can’t connect to the barcode reader. |

## MotoSetting v1.0.2 Tool

Start screen of the tool.

1. 2. 3. 4. 5. 6.

- Menu bar - PULL => start scan - RELEASE => stop scan - Clear => clear status window - R => Restore default parameters - Status window

| Symbology |  | Description and what settings are available |
| --- | --- | --- |
| UPC/EAN |  |  |
| Code 128 |  |  |
| Code 39 |  |  |
| Code 93 |  |  |
| Code 11 |  |  |
| Interleaved 2 of 5 |  |  |
| Codabar |  |  |
| MSI |  |  |
| Chinese 2 of 5 |  |  |
| Matrix 2 of 5 |  |  |
| Korean 3 of 5 |  |  |
| Postal Codes |  |  |
| GS1 DataBar |  |  |
| Composite |  |  |
| 2D Symbologies |  |  |
| Macro PDF |  |  |
|  | The checkboxes in the screenshots show the default settings of the scan engine. |  |
|  | The tool automatically stores any change. |  |

# Configuration samples for HotTab

## FNC1

Every GS1-128 barcode begins with the (FNC1) “Function Code One” character and an Application Identifier (AI) number. The AI defines a specific purpose for the data field after it. The combination of the AI and its data field is referred to as the element string. The AI is represented with parentheses around it in the Human Readable representation below the barcode. Multiple element strings may also be encoded according to GS-1 specifications. Variable element strings should be encoded last in the symbol and FNC1 characters must be encoded between two or more variable length element strings.

| Deactivated: FNC1 is not shown/transmitted Activated: FNC1 is shown/transmitted |  |
| --- | --- |
| Example: (deactivated) 24000018461.0011044455432156926 Example: (activated) 24000018461.001<GS>104445543<GS>2156926 |  |

## Replace characters

In HotTab tool => Settings menu you can setup so that defined characters are replaced.

| Open - HotTab - Barcode - Settings Regex Substitution: Type in the character what you need to replace. Replace with: Type in the character what you defined as replacement. |  |
| --- | --- |
| Example: (no replacement setup) 24000018461.001<GS>104445543<GS>2156926 Example: (<GS> replaced by #) 24000018461.001#104445543#2156926 |  |

## Prefix & Suffix

In HotTab tool => Settings menu you can setup self-defined characters as prefix and/or suffix or a defined suffix.

| Open - HotTab - Barcode - Settings Prefix: Type in the character what you need as prefix in front of your barcode data Suffix: Type in the character what you need as suffix behind of your barcode data Append Enter: Set an Enter <CR> as suffix. Append Second Enter: Activate together with “Append ENTER” to have two Enter <CR> as suffix. Append TAB: Set an TAB as suffix. |  |
| --- | --- |
| Example 1: Prefix: # Suffix: two ENTER <CR> | Setting CR = Carriage Return |
| Result: #TestCode128<CR><CR> |  |

| Example 2: Suffix: ENTER <CR>+LineFeed<LF> | Setting #$ = enable function keys $# = disable function keys CR = Carriage Return LF = Line Feed |
| --- | --- |
| Result: TestCode128<CR><LF> |  |
| Example 3: Suffix: Function key F4 | Setting #$ = enable function keys $# = disable function keys F4 = Function key F4 (possible is F1 to F24) |
| Result: TestCode128<F4> |  |